- Partner ACS Telephone System Features and Codes

| System Date (#101) sets the date on the the system. |  |
| --- | --- |
| System Time (#103) changes the time on the the system. |  |
| Assigning Lines- Number of Lines (#104) determines the number of outside lines assigned to all system extensions |  |
| Transfer Return Rings (#105) programs the number of times a transferred call rings before it returns to an extension |  |
| Recall Timer Duration (#107) changes the length of the timed signal, or switchhook flash |  |
| Rotary Dialing Timeout (#108) changes the length of the Rotary Dialing Timeout |  |
| Outside Conference Denial (#109) determines if users can include more than one outside party in a conference call |  |
| Automatic System Answer Delay (#110) specifies the number of times an incoming call should ring before it is answered |  |
| Automatic System Answer Button (#111) helps the operator answer calls during busy periods |  |
| Direct Extension Dial Delay (#112) specifies the number of times an incoming call should ring before it is answered |  |
| Direct Extension Dial Button (#113) permits an outside caller to dial an extension or Hunt Group directly |  |
| Outgoing Call Restriction Button (#114) changes the current Outgoing Call Restriction setting for an extension |  |
| Wake Up Service Button (#115) makes an intercom call to the target extension at the scheduled time |  |
| Ring on Transfer (#119) specifies what callers hear while they are being transferred, music on hold or ringing |  |
| Automatic System Answer Mode (#121) determines how the system handles ASA calls after the greeting plays |  |
| Caller ID Type (#122) Set this feature for Singapore or USA operation |  |
| Backup Programming Automatic(#123) Backup Manual (#124) when active, the system backs up at 02:00 on the first of the month |  |
| Restore Programming (#125) reinstate system and telephone programming to the settings saved at the last system backup |  |
| Automatic Daylight/Standard Times (#126) automatically updates the system clock for annual Daylight Savings Time |  |
| Hold Reminder Tone (#127) sets the time interval at which the system will provide a reminder tone for a call on hold |  |
| Dial Mode (#201) identifies individual phone lines as touch-tone or rotary |  |
| Hold Disconnect Time (#203) changes the hold disconnect time for an outside line |  |
| Automatic System Answer Lines (#204) identifies the lines on which should be answered by Automatic System Answer |  |
| Direct Extension Dial Lines (#205) identifies the lines on which incoming calls are to be answered for direct extension dial |  |
| Group Call Distribution (#206) allows calls to ring directly into a Hunt Group instead of being answered by the receptionist |  |
| Pool Line Assignment (#207) assigns or removes lines from the main and auxiliary pools supported for Hybrid mode |  |
| Line Coverage Extension (#208) identifies an extension with a specific line, enabling Call Coverage or VMS Cover |  |
| Unique Line Ringing (#209) assigns a ringing pattern to differentiate, by sound, which line is ringing |  |
| Line Assignment (#301) assigns the outside lines that are available at an extension |  |
| Line Access Restriction (#302) blocks an extension from receiving and/or making outside calls on a specific line |  |
| Display Language English/Spanish/French (#303) changes the language in which display messages appear |  |
| Automatic Extension Privacy (#304) prevents a user from joining an active call at an extension |  |
| Abbreviated Ringing (#305) prevents incoming calls from distracting you when you are busy on another call |  |
| Transfer Return Extension (#306) identifies a different return extension for an unanswered call |  |
| Forced Account Code Entry (#307) identifies extensions where users must enter an account code before making outside calls |  |
| Distinctive Ring (#308) determines whether calls should ring at a phone using a distinctive ringing pattern |  |
| Intercom Dial Tone (#309) determines the type of dial tone that the system provides at an extension |  |
| Automatic VMS Cover- direct to voice mail (#310) automatically routes an extension’s calls to the Call Answer Service |  |
| External Hotline (#311) When the handset of an external hotline is lifted, a predetermined outside number is dialed |  |
| Voice Interrupt On Busy (#312) identifies extensions that can receive Voice Interrupt On Busy calls |  |
| Line Access Mode (#313) designates extensions for key mode even though the system is configured for Hybrid mode |  |
| Pool Extension Assignment (#314) identifies the pools that are assigned to buttons at a pooled extension |  |
| Pool Access Restriction (#315) restricts a pooled extension from all lines belonging to a specific pool |  |
| Call Waiting (#316) is distinct from the local telephone company’s call waiting feature. |  |
| Caller ID Log Answered Calls (#317) helps to track callers’ telephone numbers |  |
| Caller ID Call Log Line Association (#318) associates lines with extensions for logging unanswered Caller ID calls |  |
| Caller ID Log All Calls (#319) programs one extension to log all calls received at any extension on specific lines |  |
| Call Coverage Rings (#116 or #320) defines the number of times call ring before they are sent to the covering extension |  |
| VMS Cover Rings (321#) defines the number of times a call rings before it is sent to a user’s voice mailbox |  |
| Remote Call Forwarding (#322) allows all calls to be forwarded from your extension to an outside number |  |
| Copy Settings (#399) |  |
| Outgoing Call Restriction (#401) specifies the types of outgoing calls that can be made on all lines at an extension |  |
| Toll Call Prefix (#402) indicates whether users must dial a 0 or 1 before the area code to make a long distance call |  |
| System Password (#403) defines a four-digit password to override dialing restrictions |  |
| Disallowed Phone Number Lists (#404) specify telephone numbers that users cannot dial |  |
| Disallowed List Assignments (#405) assigns up to eight disallowed phone number lists to specific extensions |  |
| Emergency Phone Number List (#406) up to 10 telephone numbers that all users can dial regardless of dialing restrictions |  |
| Allowed Phone Number Lists (#407) numbers that users can dial regardless of other dialing restrictions |  |
| Allowed List Assignments (#408) assign up to eight Allowed Phone Number Lists to a specific extension |  |
| Forced Account Code List (#409) ensures that only authorized users with valid account codes can make outside calls |  |
| Star Code Dial Delay (#410) forces a delay in Auto Dialed calls so that the system waits until the second dial tone begins |  |
| Pickup Group Extensions (#501) assign extensions to a pickup group which can answer ringing calls by dialing the group code |  |
| Calling Group Extensions (#502) place extensions in a group that can be called at the same time |  |
| Night Service Button (#503) programs Night Service Group user phones to ring immediately |  |
| Night Service Group Extensions (#504) assigns extensions to the Night Service Group |  |
| Hunt Group Extensions (#505) assign extensions to a Hunt Group where incoming calls search for the first available extension |  |
| Fax CNG Detection (#505, Group 8) allows voice calls and fax calls to be received on the same line |  |
| VMS Hunt Delay (#506) determines when outside calls should be answered by the Automated Attendant |  |
| VMS Hunt Schedule (#507) determines whether outside lines hunt all the time, during day operation or only at night |  |
| Fax Machine Extensions (#601) identifies the extensions to which fax machines are connected |  |
| Music-On-Hold (#602) activates or deactivates the Music-On-Hold jack on the processor module |  |
| Hotline (#603) when a user lifts the handset of the hotline telephone, the alert extension rings |  |
| Doorphone Extension (#604 & #605) identifies an extension to which a doorphone is connected |  |
| Doorphone Alert Extensions (#606) identifies extensions that are signaled when a doorphone button is pressed |  |
| Automated Attendant Extensions (#607) identifies extensions where auto attendants are connected |  |
| SMDR Record Type (#608) specifies whether all calls or outgoing calls only are to be included on call reports |  |
| SMDR Top of Page (#609) notifies the system that the printer has been aligned to the top of a new page |  |
| SMDR Output Format (#610) specifies whether 15 or 24 digits is printed in the Number field on the call report |  |
| SMDR Talk Time (#611) indicates whether or not you want to include a Talk field on call reports |  |
| Contact Closure Group (#612) specifies which extensions can activate each contact closure/ door buzzer |  |
| Contact Closure Operation Type (#613) specifies how long the contact closures remain open when they are activated |  |
| Music-On-Hold Volume (#614) select one of seven volume settings for the external music source |  |
| Loudspeaker Paging Line (#617) specifies the line jack to which you connected the loudspeaker paging system |  |
| System Reset–Programming Saved (#728) resets the system while retaining the currently programmed settings |  |
| Remote Administration Password (#730) creates a password for remote or local PC administration of the system |  |