Avaya 355a adapter pinout

Used for Marlin Magix programming or for SMDR

EIA/TIA-561 Pin Layout and RS-232 pin assignments

| 8P8C (RJ45) | DB9 | DB25 | Signal | Description |
| --- | --- | --- | --- | --- |
| 1 | 9 | 22 | RI | Ring Indicator |
| 2 | 1 | 8 | DCD | Data Carrier Detect |
| 3 | 4 | 20 | DTR | Data Terminal Ready |
| 4 | 5 | 7 | SG | Signal Ground |
| 5 | 2 | 3 | RXD | Received Data |
| 6 | 3 | 2 | TXD | Transmitted Data |
| 7 | 8 | 5 | CTS | Clear To Send |
| 8 | 7 | 4 | RTS | Request To Send |