## Flash Voice Mail

Please choose one of the following help TOPICS:

Topic: System Settings

- Enabling/Disabling the General Delivery Mailbox

- How to determine your Voice Mail System EXTENSION NUMBER

Topic: Automated Attendant Setup

- Recording the Day & Night Greetings

- Automated Attendant Line Answering Settings

Topic: Custom Call Routing Setup

- BUILDING and Recording the HOME Menu

- CHANGING the Home Menu

- Programming the Dialing Options

- Disabling the CCR Tree

- Enabling the CCR Tree

Topic: Mail Box Administration

- Adding a New Mail Box

- Deleting a Mail Box

- Resetting a Mail Box Password

- Changing the Name on a Mail Box

Topic: Personal Mail Box Settings

- Recording your name in the company directory

- Changing your Voice Mailbox password

- Programming Remote Notification

=======================================

How to determine your Voice Mail System  EXTENSION NUMBER

Feature 985

The Display will read: "Set : ###"

=======================================

Enabling/Disabling the General Delivery Mailbox

Feature 983

LOG: 120000 (2 digit ext's) or 1020000 (3 digit ext's) or 10020000 (4 digit ext's)

Press OK

Display will read: "ADMIN"

(3) Softkeys are:    MBOX    AA    OTHR

Press "6"

Display will read: "Gen Delivery: Y or N"

Press the CHANGE softkey

Press OK softkey

Press RLS Key.

=========================================

To Record the Company DAY & NIGHTGreetings:

Feature 983

LOG: 120000 (2 digit ext's) or 1020000 (3 digit ext's) or 10020000 (4 digit ext's)

Press OK

Press AA

Press GRTG

Press GRTG

<Display> Greeting:

Enter 1 for DAY Greeting --- Enter 2 for NIGHT greeting

Press OK

Press REC to record.

Wait for the tone!

Record your basic DAY Greeting, for example:

Thank you for calling <Company Name>. If you know your Party’s extension, you may     dial it at any time or press # for the directory. To reach the Operator, press Zero.

(Do Not record the Directory Options at this time.)

# OR

Record your basic NIGHT Greeting, for example:

Thank you for calling <Company Name>. Our Office hours are Monday through Friday,     8:00 am to 5:30 pm Pacific Time. If you know your party’s extension, you may dial it at any time or press # for the directory. To reach our General Delivery Mailbox, please press Zero.

(Do Not record the Directory Options at this time.)

============================================

Building and Recording the HOME Menu:

Feature 983

LOG: 120000 (2 digit ext's) or 1020000 (3 digit ext's) or 10020000 (4 digit ext's)

Press OK

Press OTHR

Press CCR

Press ADMIN

CCR Tree:

Enter 1

Press OK

BUILD NEW TREE?

Press YES

PATH:

Enter 0 (zero)

Display: Home/Menu

Press REC (Note: This is where you record your options. For example:

For Tom Smith, please press 1

For Bill Thomas, please press 2

For Sally Helms. Press 3

(You can have up to 8 options per branch.)

After programming each branch press END, then press SAVE

When it asks to “SAVE AS TREE 1” press YES

=============================================

CHANGING the HOME Menu:

Feature 983

LOG: 120000 (2 digit ext's) or 1020000 (3 digit ext's) or 10020000 (4 digit ext's)

Press OK

Press OTHR

Press CCR

Press ADMIN

CCR Tree:

Enter 1

Press OK

Press CHG

PATH:

Enter 0 (zero)

Display: Home/Menu

Press REC

Display: Primary REC?

Press: REC (Note: This is where you record your options). For example:

For Tom Smith, please press 1

For Bill Thomas, please press 2

For Sally Helms. Press 3

Press OK after you finish recording

Display: Alternate REC?

Press OK

Press END, then press SAVE

When it asks to “SAVE AS TREE 1” press YES

===============================================

Programming the Dialing Options:

Feature 983

Log: 120000 (2 digit ext's) or 1020000 (3 digit ext's) or 10020000 (4 digit ext's)

Press: OTHR

Press: CCR

Press ADMIN

CCR Tree:

Enter 1

Press OK

### Press CHG

PATH:

Enter 1 or 2 or 3 or 4 or 5 etc.

Press  1

Press OTHR

Press XFER

PRESS INT

Ext: <Enter the extension number for option ‘1’> For example, if you recorded Tom Smith as option 1, program Tom Smith’s extension number here.

Repeat these steps for options 2 thru 8. Note: you can have as many as 8 options on the main branch. You can have as few as 1.

NEVER EXIT PROGRAMMING WITHOUT SAVING YOUR CHANGES!!!!

To Do so, Press END, SAVE (Save as tree 1?) YES

Press the RELEASE KEY

=====================================================

Disabling the CCR Tree:

Feature 983

Log: 120000 (2 digit ext's) or 1020000 (3 digit ext's) or 10020000 (4 digit ext's)

Press AA

Press TABLE

Table:

Enter 1

Press OK

Press Next until you see <CCR Tree:>

Press CHG

Press DISABLE-

Display should read “Tree Disabled”

Press the RELEASE Key

===================================================

Enabling the CCR Tree:

Feature 983

LOG: 120000 (2 digit ext's) or 1020000 (3 digit ext's) or 10020000 (4 digit ext's)

Press OK

Press AA

Press TABLE

Table:

Enter 1

Press OK

Press Next until you see <CCR Tree:>

Press CHG

Enter 1

Display should read “Tree Enabled”

Press OK

Press the RELEASE Key

===================================================

Recording your name in the Company Directory:

Feature 981

Pswd: enter your personal password

Press OK

Press ADMIN

Press the # 1 key

Your mailbox will then broadcast the EXISTING recording.

You will then hear a Beep.

<Display> Recording:

SAY YOUR FIRST AND LAST NAME

Press OK

Your name has now been recorded in the Directory!